EYE ON YOU

MVP TECHNICAL SPECIFICATION & REQUEST FOR QUOTATION

Track A — Adaptive Tint Smart Eyewear (excludes electronically variable prescription optics)

CONFIDENTIAL This document contains proprietary product information belonging to the Founder. It is provided to prospective manufacturing partners solely for the purpose of quotation and feasibility assessment, and is subject to the Non-Disclosure, Non-Use, and Non-Circumvention Agreement executed between the parties.

# 1. Product Overview

EYE ON YOU is a premium adaptive eyewear product that automatically adjusts lens tint and comfort settings based on ambient conditions, with companion mobile app control. This specification covers the Track A MVP scope only: sensor-driven adaptive tint, a passive/replaceable optical insert for prescription (ground by a licensed optician, not adjusted electronically), motion and light sensing, Bluetooth connectivity, and app-controlled modes. It does not include an electronically variable prescription lens — that capability is a separate, longer-term R&D track and is out of scope for this quotation.

# 2. Scope of This RFQ

- Frame: one silhouette for initial sampling (Classic Rectangular), unisex sizing

- Lens: electrochromic or photochromic tint layer, clear-to-dark transmission range, laminated over a passive corrective or plano base lens

- Electronics: ambient light sensor, IMU, BLE module, MCU, battery, charging contacts, status LED, physical button

- Charging case: compact case with battery, USB-C input, magnetic charging contacts for the glasses

- Firmware: basic sensor-to-tint control loop and BLE communication with a companion app (app itself developed separately by Founder's team)

# 3. Bill of Materials / Component Specification

| Component | Specification | Notes |
| --- | --- | --- |
| Frame | Titanium alloy or bio-acetate; target weight 26–31g complete | Classic Rectangular style for first sample |
| Lens — tint layer | Electrochromic or high-quality photochromic film; clear (~90% transmission) to dark (~15% transmission) | Must not require a visible driver chip inside the lens field of view |
| Lens — base optical layer | Passive plano or single-vision ground lens, standard optical-grade polycarbonate or CR-39 | Prescription glazing done by optician post-manufacture, not by factory |
| Ambient light sensor | Digital ALS, I2C interface, lux range 1–65,000 | Mounted at upper lens edge or bridge, unobtrusive |
| Motion sensor (IMU) | 6-axis accelerometer + gyroscope, low-power mode | Used for wear detection and basic activity sensing |
| Connectivity | Bluetooth Low Energy 5.3, standard GATT profile | Must support custom service/characteristic for app control |
| Processor | Low-power ARM Cortex-M0/M4 class MCU | Sufficient for sensor fusion + BLE stack |
| Battery | Li-polymer, target 35–40mAh per temple | UN38.3 certified cells required for air freight |
| Charging | Magnetic pogo-pin contacts, 2–4 pin | Auto-aligning with case dock |
| Controls | One physical push button minimum | Capacitive touch zone optional, secondary |
| Status LED | Single color, <2mm visible aperture | Brightness should scale with ambient light |
| Nose pads | Replaceable silicone, 3 sizes | Standard eyewear industry mounting |
| Water resistance | IPX4 minimum | Light rain / sweat resistance, not submersion |
| Charging case | Aluminum or polycarbonate shell, capacity ≥1000mAh | USB-C input; Qi wireless input is a nice-to-have, not required for sample |

# 4. Certifications Required Before Sale

- CE marking (EU) and FCC certification (US) for the electronic module

- RoHS compliance for all materials

- UN38.3 test report for lithium battery cells (required for air shipment)

Founder is separately assessing whether marketing claims trigger medical-device regulatory review (FDA/MDR); this is not a manufacturing certification and is outside the scope of this RFQ.

# 5. Requested Quotation Detail

| Component | Specification | Notes |
| --- | --- | --- |
| Sample / prototype run | 5–10 units, all components as specified | Please quote unit cost, tooling cost (if any), and lead time |
| Pilot production run | 500–1,000 units | Please quote unit cost at this volume and lead time |
| Mass production run | 5,000+ units (indicative only) | Please provide indicative pricing and MOQ tiers |

# 6. Information Requested From Manufacturer

- Company name, business license number, and years in operation

- Prior experience manufacturing smart eyewear, electrochromic/photochromic lenses, or similar wearable electronics (references or case studies welcome)

- Full itemized quotation per Section 5, including tooling/mold costs if applicable

- Sample lead time and mass production lead time

- Willingness to sign the accompanying NNN Agreement prior to receiving detailed design files

- Applicable certifications already held (ISO 9001, CE, RoHS testing capability, etc.)

# 7. Contact

Company/Founder name: ____________________________

Email: ____________________________

Preferred contact method / time zone: ____________________________